Southern Impressions Homes

10151 Deerwood Park Blvd, Building 400, Suite 300, Jacksonville, FL 32256

904-517-5939

https://www.southernimpressionhomes.com/

Congratulations on moving into your Southern Impressions Home!  Whether your home is professionally managed by our partner, Suncoast Property Management, another property management company, or you are the owner occupying the home, our warranty is there for you.  A copy can be found here for reference 2-10-Home-Buyers-Warranty.pdf (southernimpressionhomes.com).

If you experience an issue and your property is managed by Suncoast Property Management or another property management company, please contact them for assistance.

If you are a homeowner or a Property Manager, please place all warranty requests through our website at Warranty Form | Southern Impression Homes.  Someone will contact you on your request within 72 hours.

For your information, we have attached a few forms to assist you with your new Southern Impressions home.  You will find some new home care tips, troubleshooting tips, and forms to record serial numbers and locations of important items.  There is also a form with contact numbers.

Once again, we would like to thank you for trusting us in delivering your new home.

Sincerely,

Southern Impressions Homes

Southern Impressions Homes

10151 Deerwood Park Blvd, Building 400, Suite 300, Jacksonville, FL 32256

904-517-5939

https://www.southernimpressionhomes.com/

NEW HOME CARE TIPS

- Store your instruction manuals and warranties in a safe, easy-to-find place.

- Mail in all manufacturer warranties on new systems and appliances.

- Record Model Numbers and Serial Numbers for quick reference.

- Locate the main water and gas shut-off valves.

- Locate the sewer/septic clean out and lift station control box if applicable.

- Locate the electrical service panel and shut-off switch.

- Get to know your appliances and how they work.

- Change your HVAC filter monthly.

Southern Impressions Homes

10151 Deerwood Park Blvd, Building 400, Suite 300, Jacksonville, FL 32256

904-517-5939

https://www.southernimpressionhomes.com/

NEW HOME TROUBLESHOOTING TIPS

- When calling in work for items such as no hot water, lights or receptacles not working, please check the circuit breakers in the panel.  In the case of kitchen, bathroom, and exterior receptacles not working check to see if the GFCI has tripped and reset it.

- Circuit breakers are safety devices in the home.  New electrical codes have required some items that one may not be familiar with.  Circuits for lighting and receptacles contain an AFCI or Arc Fault Circuit Interrupter.  While they have gotten better over the years, they can trip easily and for no apparent reason.  They can easily be reset.

- New electrical codes have also introduced tamper resistant receptacles.  When they are new, they can be difficult to insert a plug into it.  Make sure the tines on the plug are straight and they are being inserted evenly.

- Circuit breakers have an amperage rating.  Exceeding that rating can cause the breaker to trip.  Kitchens and bathrooms typically have 20 amp breakers.  In a bathroom, that allows for a blow dryer (10-12 amps) to be used.  Normally all bathrooms in the home are on the same breaker.  Using two blow dryers at the same time may cause the breaker to trip.  In the kitchen, running a coffee maker (10 amps) and a toaster oven (15 amps) on the same circuit at the same time can cause the 20 amp breaker to trip.  This is not a defect.

- HVAC systems are now required to meet higher efficiency standards and unit design is based on a set of industry standard calculations.  These systems are designed to heat to an indoor temperature of 70 degrees at 41 degrees outside, and cool to a temperature of 75 degrees at 91 degrees outside.  The system may heat or cool to those indoor temperatures beyond the design, but it is not guaranteed.

- All new toilets are required to be low flush and use between 1.28 and 1.6 gallons of water.  Dual flush toilets use about .88 gallons on the lower flush mode.  This low amount of water typically does not cause issues.  Flushing excessive amounts down the toilet may affect the operation.

- You may experience lower water flow through lavatory faucets and showerheads.  The EPA has mandated that lavatory faucets provide no more than 1.5 gallons per minute and showerheads provide no more than 2 gallons per minute.

- There may be a decrease in the water pressure or volume felt on the hot water side.  This is due heat traps being installed on the water heater.  These heat traps prevent the hot water from migration outside of the water heater when not in use.  Experiencing a lower pressure on the hot side is not a defect.

- Water heaters are factors set at 120 degrees Fahrenheit.  That is hot enough for domestic needs and protects from scalding.  Due to liability issues, water heater temperatures cannot be adjusted.

- There are many products on the market that state that they are flushable.  They will flush but may cause significant damage to domestic sewer systems and septic systems.  Many septic systems now have a macerator pump which grinds waste.  Products such as flushable wipes and feminine products over time will cause pump failure because the pump cannot grind them.  Additionally, these products along with food waste do not biodegrade and will cause a septic system to fill up faster than normal.   It is recommended that these types of products not be used.

- A sulfur/rotten egg smell in the hot water is not uncommon initially in the home.  Water sitting in the tank causes a reaction inside the water heater and creates the odor.  Flush the hot water through all faucets and tubs for about 30 minutes to replace the water.

Southern Impressions Homes

10151 Deerwood Park Blvd, Building 400, Suite 300, Jacksonville, FL 32256

904-517-5939

https://www.southernimpressionhomes.com/

MAJOR APPLIANCE INFORMATION

| APPLIANCE | MANUFACTURER | MODEL NUMBER | SERIAL NUMBER |
| --- | --- | --- | --- |
| Air Handler (Inside) |  |  |  |
| Condensing Unit (Outside) |  |  |  |
| Well Pump |  |  |  |
| Water Treatment System |  |  |  |
| Range |  |  |  |
| Refrigerator |  |  |  |
| Range Hood/Microwave |  |  |  |
| Dishwasher |  |  |  |
| Water Heater |  |  |  |
| Washing Machine |  |  |  |
| Dryer |  |  |  |

NOTE:  Not all homes will have every appliance listed.

Southern Impressions Homes

10151 Deerwood Park Blvd, Building 400, Suite 300, Jacksonville, FL 32256

904-517-5939

https://www.southernimpressionhomes.com/

CONTACT INFORMATION

| CONTACTS | NAME | PHONE NUMBER |
| --- | --- | --- |
| Property Manager |  |  |
| HVAC |  |  |
| Plumber |  |  |
| Well |  |  |
| Septic |  |  |
| Electrician |  |  |
| Electrical Provider |  |  |
| Water Provider |  |  |
| Appliances |  |  |

NOTE:  If you are a tenant, please contact your Property Management Company prior to reaching out directly to a vendor.  Failure to notify your Property Manager may result in a violation of the terms of your lease agreement and you may be responsible for the cost of any repairs.

Southern Impressions Homes

10151 Deerwood Park Blvd, Building 400, Suite 300, Jacksonville, FL 32256

904-517-5939

https://www.southernimpressionhomes.com/

IMPORTANT LOCATIONS

| ITEM | LOCATION |
| --- | --- |
| Water Shutoff |  |
| Well Pump GFCI |  |
| Sewer/Septic Cleanout |  |
| Lift Station Control |  |
| Lift Station GFCI |  |
| Breaker Panel |  |
| Main Breaker |  |
| Exterior Receptacle GFCI |  |
| Bathroom Receptacle GFCI |  |
| Kitchen Receptacle GFCI |  |
| Gas Shutoff |  |
| HVAC Filter Size |  |

NOTE:  Not all homes will have every item listed.